Physics Required Practical – Resistance series and parallel circuits

Aim:   Investigate the effect of combining resistors in series and parallel

PRACTICAL SKILLS:

| Good: I can record all my data to the accuracy of the measuring instrument and calculate resistance. |  |
| --- | --- |
| Great: I can analyse data to make informed predictions. |  |
| Even better: I can analyse data and formulate generic rules for calculating total resistance. I can apply my knowledge to exam questions. |  |
| Ultra Challenge: I can relate my findings to springs in series and parallel. |  |

EQUIPMENT

You have access to the following:

- a battery or suitable power supply

- ammeter or multimeter

- voltmeter or multimeter

- crocodile clips

- two 10Ω resistors

- connecting leads.

DIAGRAMS

Series								Parallel

TASK 1 (Series)

- Connect the circuit for two resistors in series, as shown in the diagram.

- Switch on and record the readings on the ammeter and the voltmeter.

- Use these readings to calculate the total resistance of the circuit.

| V / V | I / A | R / Ω |
| --- | --- | --- |

- With one single resistor in the circuit, the total resistance would be 10 ohms.  What is the effect on the total resistance of adding another identical resistor in series?

……………………………………………………………………………………………………………………………………………………………………………………………………

- a. Predict what the resistance will be if had three resistors in series.

……………………………………………………………………………………………………………………………………………………………………………………………………

- Now use your equipment to find the resistance with three resistors in series.

| V / V | I / A | R / Ω |
| --- | --- | --- |

- Was your prediction right? ……………………………………………………………………

- Analyse your results to create a generic rule and formula for calculating the total resistance when resistors are joined in series.

……………………………………………………………………………………………………………………………………………………………………………………………………

……………………………………………………………………………………………………………………………………………………………………………………………………

TASK 2 (Parallel)

- Now set up the circuit for two resistors in parallel (see diagram).

- Switch on and record the readings on the ammeter and the voltmeter.

- Use these readings to calculate the total resistance of the circuit.

| V / V | I / A | R / Ω |
| --- | --- | --- |

- With one single resistor in the circuit, the total resistance would be 10 ohms.  What is the effect on the total resistance of adding another identical resistor in parallel?

……………………………………………………………………………………………………………………………………………………………………………………………………

- a. Predict what the resistance will be if had three resistors in parallel.

……………………………………………………………………………………………………………………………………………………………………………………………………

- Now use your equipment to find the resistance with three resistors in parallel.

| V / V | I / A | R / Ω |
| --- | --- | --- |

- Was your prediction right? ……………………………………………………………………

- Analyse your results to create a generic rule and formula for calculating the total resistance when resistors are joined in parallel.

……………………………………………………………………………………………………………………………………………………………………………………………………

……………………………………………………………………………………………………………………………………………………………………………………………………

Applying your knowledge

###### (a)     A student is given three resistors of resistance 3.0 Ω, 4.0 Ω and 6.0 Ω respectively.

###### (i)      Draw the arrangement, using all three resistors, which will give the largest resistance.

###### (ii)     Calculate the resistance of the arrangement you have drawn.

###### (iii)     Draw the arrangement, using all three resistors, which will give the smallest resistance.

###### (iv)    Calculate the resistance of the arrangement you have drawn.

###### (5)

###### (b)     The three resistors are now connected to a battery of emf 12 V and negligible internal resistance, as shown in Figure 1.

###### Figure 1

###### (i)      Calculate the total resistance in the circuit.

###### .............................................................................................................

###### .............................................................................................................

###### (2)

###### (Total 7 marks)

Ultra Challenge (beyond this experiment …)

- Three identical resistors joined in parallel in an electrical circuit share the total current in the circuit. In a similar way, the three springs in the chest expander share the total force exerted. The spring constant of one spring is 400N/m.

###### By considering this similarity, determine the total force exerted on the chest expander when each spring is stretched by 0.25 m.

###### ........................................................................................................................................................

###### ........................................................................................................................................................

- When adding springs in series the total spring constant takes the same rule as when adding resistors in parallel. The formulas are shown below.

###### Figure 1                                               Figure 2

###### When a 200 g mass is suspended from a spring, as in Figure 1it is calculated that the spring constant is 56N/m.  A spring identical to that in part (a) is joined to the lower end of the original one and a 500 g mass is suspended from the combination, as shown in Figure 2.

###### State the value of the new spring constant for this combination of two springs.

###### .........................................................................................................................

- When adding springs in series the total spring constant takes the same rule as when adding resistors in parallel. The formulas are shown below.

###### A load of 4.0 N is suspended from a parallel two-spring system as shown in the diagram.

###### The spring constant of each spring is 20 N m–1. The elastic energy, in J, stored in the system is

Hint:

What is Hooke’s Law equation?

What is the equation for the elastic energy stored by a spring?

|  | A | 0.1 |
| --- | --- | --- |
|  | B | 0.2 |
|  | C | 0.4 |
|  | D | 0.8 |